### Before Inspection

- Run engine until warm.

- Check operation of starter, vacuum pump, alternator, magnetos, carb heat, propeller, oil temp and pressure, fuel pressure, and other accessories and instrument functionality including radios, gps, vor, etc.

### Airframe

- General Condition Inspection

  - Check for dings, dents, loose/broken rivets, and cracks that must be repaired because of their nature.

  - Check for current damage.

  - Check for obvious signs of previous damage.

  - Check for internal corrosion, especially around gear box.

  - Check control cables for correct tension.

  - Check for fuel leaks on upper and lower wing and fuselage surfaces.

  - Check for leaks around fuel selector (move to different positions and let sit at each one for a while)

  - Check controls (including trim tabs, flaps, and cowl flaps) for alignment, rigging, full travel, freedom of movement and security.

  - Inspect cabin and exterior for signs of water leakage and damage, especially around windshield.

  - Check cowling for fit and security. Note any repair work needed for cracks.

  - Check cabin and baggage doors for correct fit.

  - Check cabin and baggage doors for hinge wear.

  - Check tires and brakes for security and wear.

  - Check glass for cracks, crazing, and scratches.

  - Check all antennas for correct installation and security.

  - Check pitot heat. Check pitot tube and static sources for obstructions.

  - Check fuel vents for obstructions

  - Check fuel caps for tightness and seal

  - Check fuel placards for correct type of fuel and quantity.

  - Check all external and internal lights for operation.

  - Check seat belts and harnesses (if installed) for wear.

  - Check seats for correct movement and that proper stops are installed.

  - Check that compass card is installed.

  - Check condition of plastic interior trim.

  - Check that weight and balance, equipment list, and airworthiness certificate are present and current.

  - Look inside fuel tanks for tools, debris, etc.

  - Check fire extinguisher.

- Inspect the belly for possible damage or excessive exhaust residue.

- Check the ELT for operation and current battery.

- Check battery box for corrosion. Check battery for proper charge.

- Check under instrument panel for proper avionics and equipment wiring -- any fuses or all circuit breakers?  Modern wiring or original?

- Check circuit breaker panel for correct markings.

### Engine

- Check for oil leaks.

- Check for evidence of exhaust leaks.

- Check muffler for evidence of cracks or wear.

- Check compression using differential method. Be sure intake valves move (I bought a plane once with a push rod missing -- not broken, just not installed)

- Check alternator belt for security and wear.

- Take oil sample and send to lab. Cut oil filter open and check for unusual metal or other contamination.   They just changed the oil, so this may not be valuable.

- Check security of all accessories. Check them for case cracks.

- Check cylinders for cracks in known locations and around spark plug bosses.

- Check engine case for cracks.

- Check oil breather system for obstructions.

- Check condition of ignition harness.

- Check engine mount for cracking and lord mount wear.

- Check condition of firewall, especially near gear boxes.

- Check condition of electrical wiring forward of the firewall and as it goes through to the cabin.

- Check propeller for nicks and/or leaks.

- Check operation of all mechanical controls.

- Check ignition switch for proper operation. Check for shorts in mags by prop starting with mags off.

### Logbooks

- Research all AD notes.

- Check service bulletin compliance. Make recommendation for complying with significant ones that have not been done.

- Check logbooks for damage history and completeness.

- Check annual inspection entry for proper signoff.

- Check for IFR recertification.

- Compare engine, airframe, and propeller serial numbers with those in logbooks.

- Verify engine and propeller times since major overhaul and verify that engine overhaul log entry is complete and valid.

- Check age of bladder fuel tanks (if installed).

- Determine Part 135 compliance

- Check that the POH is correct for that aircraft